Antenna Composite Pattern Test Report

| Manufacturer | JAGUAR WAVE |
| --- | --- |
| Sample Received | Feb.02,2024 |
| Start Test Date | Feb.02,2024 |
| Final Test Date | Feb.02,2024 |

JAGUAR WAVE Technology Co,Ltd. Shenzhen Laboratory

14F; TINNO HQ Building, Tongfa South Rd, Xili, Nanshan District, Shenzhen

Table Of Contents

1. Test Frequency	4

2. Testing Location	4

3. Test Facility and Configuration	5

4. Measured Values and Calculation of Maximum Gain Positions	6

5. Test Setup	7

6. Test Results	7

History of this report

| Version | Description | Issued Date |
| --- | --- | --- |
| 01 | Horn Antenna Test | Feb.02,2024 |

- Test Frequency

| Band (GHz) | Test Frequency (GHz) |
| --- | --- |
| 75-110 | 90 |
| 75-110 | 90.5 |
| 75-110 | 91 |
| 75-110 | 91.5 |
| 75-110 | 92 |
| 75-110 | 92.5 |
| 75-110 | 93 |
| 75-110 | 93.5 |
| 75-110 | 94 |
| 75-110 | 94.5 |
| 75-110 | 95 |

- Testing Location

Testing Location

JAGUAR WAVE Technology Co,Ltd. Shenzhen Laboratory

14F; TINNO HQ Building, Tongfa South Rd, Xili, Nanshan District, Shenzhen

| Test Condition | Test Engineer | Test Environment (°C / %) | Test Date |
| --- | --- | --- | --- |
| Radiated | Ck Yang | 23-24 / 50-55 | Feb.02,2024 |

Note:

Testing Site Information

Brand Name: NSI-700S-360

Dimension: 5m*4m*3m

Characteristic: Spherical Near-Field / Far-Field Chamber

- Test Facility and Configuration

NSI-700S-360 SNF Scanner

- Spherical system for stationary mm-wave antennas

- Well suited to applications above 10 GHz.

- Near-field and Far-field

- Multi-axis (3) high accuracy stepper motor

- Large enough to incorporate RF converter modules.

- Probe tip radius of roughly 20" (500 mm).

- All positioners contain integrated RF rotary joints to maximize cable phase stability during testing

- Measured Values and Calculation of Maximum Gain Positions

max value position

| Frequency(GHz) | Gain(dBi) |
| --- | --- |
| 90 | 23.551 |
| 90.5 | 23.569 |
| 91 | 23.587 |
| 91.5 | 23.604 |
| 92 | 23.621 |
| 92.5 | 23.638 |
| 93 | 23.654 |
| 93.5 | 23.67 |
| 94 | 23.685 |
| 94.5 | 23.7 |
| 95 | 23.715 |

- Test Setup

Test Position

Antenna Pattern Coordination

Note: Photos of Test Position: Please refer to the test photos in the appendix

- Test Results

- Antenna Pattern

(XZ plane) – Θ(0 -180)Φ(0) and Θ(0-180)Φ(180)

(YZ plane) – Θ(0 -180)Φ(90) and Θ(0-180)Φ(270)

- Test Photos